Installation Guidelines

# Accessories

TEX Tread Plate/ TEX Sheet

Data Sheet:   (sheet 1 of 3) August 2013 (DT)

These installation guidelines apply to the following Gradus profiles:

TEX Tread Plate, TEX Sheet

- It should be ensured that all surfaces to which the profile is to be fixed are dry, flat and free from any contaminants (grease, oil, loose debris). Where concrete surfaces are damaged, these should be prepared with a proprietary levelling repair compound. Painted metal finishes should have any loose paint or rust removed prior to installation.  Timber treads or decking should be checked and where rot or damage has occurred, these areas should be replaced or repaired prior to TEX installation.

- It is essential that prior to installation stair treads or floor areas are checked for drainage or adequate run offs to avoid water pooling or ice build up during inclement weather.

- Ensure both the TEX Plate and TEX Sheet material are fully supported from the underside to avoid damage to the material during use.

Please note screw concealing plugs are not available with this product.

Installation of Tread Plates

TEX Flooring can be surveyed prior to installation by one of our trained surveyors and the product can be cut to size for use as tread plate or in sheet format. For any final site adjustment then the material can be trimmed using either a jig saw with a Bosch T127D HSS Blade or angle grinder with a diamond blade.  Notes: - the material is supplied with a tolerance of plus zero minus 8/10 mm to minimise any site trimming.  Tread Plates will be cut to a standard width. If the tread plate is required to be scribed to the step riser or stringer then the Tread Plate will be supplied over size [on request] and any trimming would be carried by the operative installing the material.

The substrate will determine the method of installation but all will require mechanical and adhesive combination to ensure the product is fully secured for use.

Concrete, Marble or Stone:

- The Tread Plate should be installed in conjunction with the TEX stair edging. The Tread Plate has a routered leading edge that allows the Tread Plate to lock securely into the back edge of the Stair Edging.

- The Tread Plate should be pre drilled and countersunk to accommodate No 10 stainless steel screws at 250/300 mm centres, set at 30 mm from the front edge and 50mm from the back edge of the panel. First and last fixings should be positioned 150 mm from the stringer edge of the stair.

- The Tread Plate should then be positioned by pushing tightly against the back edge of the stair edging. The screw holes should then be marked and the Tread Plate removed and holes drilled to accommodate a fixing suitable to take a No10 38mm long stainless steel screw.

- Plug fixings should then be placed in each of the pre drilled holes and a suitable proprietary cartridge adhesive, applied to the underside of the tread plate by applying a bead of adhesive around the perimeter of the panel 25mm from each edge and a zigzag bead liberally applied across the balance of the Tread Plate. A bead of proprietary mastic should be applied to the routered leading edge.

- Position the Tread Plate behind the TEX Stair Edging and secure using No 10 38mm stainless steel countersunk screws. Wipe off any excess mastic that has squeezed out between the Stair Edgings and Tread Plate. .

- Any gaps left at the back of the Tread Plate or stringer edge can then be filled using a propriety mastic which will improve the final appearance and minimise any water or dirt ingress below the tread plate.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Accessories

TEX Tread Plate/ TEX Sheet

Data Sheet:   (sheet 2 of 3) August 2013 (DT)

Metal:

- The Tread Plate should be installed in conjunction with the TEX stair edging. The Tread Plate has a routered leading edge that allows the Tread Plate to lock securely into the back edge of the Stair Edging.

- The Tread Plate should be pre drilled and countersunk to accommodate No 10 stainless steel, self tapping screws at 250/300 mm centres, set at 30 mm from the front edge and 50mm from the back edge of the panel. First and last fixings should be positioned 150 mm from the stringer edge of the stair.

- A suitable proprietary cartridge adhesive is then applied to the underside of the Tread Plate by applying a bead of adhesive around the perimeter and a zigzag bead liberally applied across the balance of the Tread Plate. A bead of proprietary mastic should be applied to the routered leading edge.

- The Tread Plate should then be positioned by pushing tightly against the back edge of the stair edging. The screw holes should then be drilled to accommodate a self tapping No10 30mm stainless steel screw. Wipe off any excess mastic that has squeezed out between the stair edgings and tread plate. Where there is risk of the tips of the screws being of risk, then these should trimmed back on the underside of the metal substrate.

- Where access to the underside of the tread can be obtained then point “4” can be replaced with stainless steel bolts, washers and locking nuts as an alternative method of fixing.

- Any gaps left at the back or stringer edge can be filled using a proprietary mastic which will improve the final appearance and minimise any water or dirt ingress below the tread.

Timber:

- The Tread Plate should be installed in conjunction with the TEX stair edging. The Tread Plate has a routered leading edge that allows the Tread Plate to lock securely into the back edge of the Stair Edging.

- The Tread Plate should be pre drilled and countersunk to accommodate a No 10 stainless steel screws at 250/300 mm centres set at 30 mm from the front edge and 50mm from the back edge of the panel. First and last fixings should be positioned 150 mm from the stringer edge of the stair.

- A suitable proprietary cartridge adhesive is then applied to the underside of the Tread Plate by applying a bead of adhesive around the perimeter and a zigzag bead liberally applied across the balance of the Tread Plate. A bead of proprietary mastic should be applied to the routered leading edge.

- The Tread Plate should then be positioned by pushing tightly against the back edge of the stair edging. The screw holes should then have small pilot hole drilled to ease the installation of the fixing of  No 10 38mm stainless steel wood screws. Wipe off any excess mastic that has squeezed out between the stair edgings and tread plate.

- Any gaps left at the back or stringer edge can be filled using propriety mastic which will improve the final appearance and minimise any water or dirt ingress below the tread.

Installation of Tread Sheet

The above fitting instructions also apply for the fitting of sheet over each of the substrates.

Additional information required.

- Sheets should be drilled at 250/300mm centres in both directions continuously across the face of the sheet and 30mm from the sheet edges.

- Sheets should have adhesive applied in a bead 25mm from the perimeter edge of the sheet, then crossed at hatched at 200 mm centres over the entire area within the perimeter bead. Dependant of the substrate the sheet it should be fixed in place using one of the methods used for fixing the tread plates

- Sheets should not be butt jointed to allow for expansion and should be fitted with 4mm gap and similar gaps left to adjacent up stands/perimeters then filled with propriety mastic.

- A ramp section is available for where the TEX Sheet meets adjoining finished floor levels and is butt jointed against the edge of the sheet. The ramp is fixed using a proprietary adhesive and the appropriate No 8 countersunk stainless steel screw fixing.

- Where the TEX Sheet meets the TEX Stair Edging at the top or half landing then the leading edge of the sheet will be routered in the same fashion as Tread Plates which will ensure the sheet is fully locked into position. The TEX Stair Edging should be fitted before the sheet is installed.

- Where stair edging cannot be fitted prior to the sheet, then measure the distance from the inside face riser of the stair edging selected for the installation to the back of the profile, less 5mm and fix the sheet this distance back from the leading edge of the stair riser.

- Note: Ensure that the sheet is installed parallel to the edge of the stair riser to ensure the correct installation of the Stair Edging. A bead of mastic is applied to the routered edge of the sheet before commencing with the fixing of the Stair Edging.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

TEX Tread Plate/ TEX Sheet

Data Sheet:   (sheet 3 of 3) August 2013 (DT)

Installation Guidelines

# Accessories

Recommended adhesive products for TEX Tread Plate/Sheet

Gradus limited provide this information only as guidance and the legal responsibility for the supply and performance of the adhesives listed below shall remain the sole responsibility of the relevant manufacturer.

By providing this information, Gradus limited does not imply any equivalents between each of the different manufacturers’ products, or that other products would prove unsatisfactory. However, use of adhesives not recommended by Gradus should be supported by a guarantee from the adhesive supplier.

Application of the named adhesive must be actioned in strict accordance with the relevant manufacturers’ instructions.

| Manufacturer | Product Reference | Recommended for | Contact number | Website |
| --- | --- | --- | --- | --- |
| F Ball | Stopgap 850 Exterior | Levelling repair compound | +44 (0) 1538 361 633 | www..f-ball.co.uk |
| Soudal | Fix All High Tack |  | 01827 261092 | www.fixall.co.uk |
| Sika Ltd | Sikaflex 11FC+ Sikaflex 221 Sikaflex 521UV |  | 01707 394444 | www.sika.co.uk |

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com