Installation Guidelines

# Accessories

EJC 75 3.2 Construction Joint Covers

Data Sheet:   (sheet 1 of 1)   28/02/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

- It should be ensured that all surfaces to which the profile is to be fixed are dry, flat and free from any contaminants.

- Cut the profile to the size required.

- Apply a proprietary contact adhesive to the underside of the profile and the floor

- Allow the adhesive to become touch dry.

- Apply the profile to the floor and press down firmly to ensure full contact with the adhesive.

- Install the resilient flooring so it abutts the profile.  EJC Construction Joint Covers can be hot welded if required.

NB. Do not install EJC Construction Joint Covers on gaps exceeding 10mm

If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428922 or fax on (+44)1625 425226