- It should be ensured that all surfaces to which the profile is to be fixed are dry, flat and free from any contaminants.

- Cut the profile to the size required.

- Apply a bead of proprietary cartridge adhesive to the undersides of the profile and firmly place into position.

Note:

This profile is designed for use with a liquid penetrant (such as with ceramic tiles or resin floors where the liquid or adhesive under the ceramics sets to a stiff solid) and therefore doesn’t warrant the need for a mechanical fix with screws.

Although there are holes in these products, these are to allow liquid or adhesive to flow through them and form an additional mechanical bond. If the depth allows for the use additional screws can be used but are not essential. Guidance for which follows on from step 2:

- Place the profile into position and through the pre-drilled holes, mark the substrate.

- Drill the marked substrate using a 5mm drill bit and once drilled insert the appropriate sized plug (5mm = yellow plug).

- Apply a bead of proprietary cartridge adhesive to the underside of the profile, then, ensuring the substrate is free from dust, firmly place the base into position.  *On timber subfloors, apply adhesive, and fix using No4 wood screws.

- Secure in place using a No4 fixing screw.

If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428 922 or

fax on (+44)1625 433 949.