MJ301SF4

Data Sheet:   (sheet 1 of 1)   27/04/10

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for MJ301SF4 is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

70 EDGINGS/COVER STRIPS / 740 EDGINGS/COVER STRIPS /

M51 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

510 EDGINGS/COVER STRIPS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433949

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: MJ301SF4

Material: Silver Anodised Aluminium

Fixing: Proprietary cartridge adhesive fix – as per manufacturers instructions

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433949

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: MJ301SF4

Material: Silver Anodised Aluminium

Gauge: 3mm

Profile Height: 4.5mm

Profile Width: 48mm

Flexible Expansion Material: 11mm

Fixing: Proprietary cartridge adhesive fix – as per manufacturers instructions

Description

Specialist Floor Trim – Movement Joint

Movement joints are designed for use where expansion of up to 1.5mm is required when joining floorcoverings.  Designed specifically for use with flooring materials such as ceramic tile, marble, stone and terrazzo.  All profiles feature a perforated base with fixing holes for straights and use with ceramics.