Installation Guidelines

# Accessories

Stainless Steel Multi-Purpose Trim

Data Sheet:   (sheet 1 of 1)   November 2013

For Ceramic Installations

These installation guidelines apply to the following Gradus profiles:

(ETR301GF8, ETR302GF8, ETR303GF8, ETR304GF8, ETR305GF8, ETR306GF8, ETR307GF8, ETR309GF8)

- A proprietary ceramic adhesive should be selected when installing with ceramic tiles.

- It should be ensured that all surfaces to which the profile is to be fixed are sound, dry, flat and free from any contaminants.

- Choose the trim height according to the thickness of the tile.  The trim should not exceed the height of the tile but should be 0.5 to 1mm lower.

- Cut the profile to the size required and allow space for linear expansion.

- Apply adhesive to the area and bed the perforated flange of the profile into the adhesive and align.

- Trowel extra adhesive over the perforations and the dovetail groove in the vertical section.

- Bed the tile into the adhesive and align with the profile leaving a gap of between 0.5 to 2mm for final grouting to be undertaken.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

11/13/DT