Cleaning and Maintenance Information

# Accessories

DLA45 Aluminium Floor Trim

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build up of dirt and grease, which will reduce the slip-resistant properties of the channel and detract from the overall appearance of the product.

NB. Under no circumstances should degreasers of any type and floor cleaners/ sanitizers containing hydrocarbons or citrus based agents be used to clean any Gradus profile.

Steam cleaners, power washers, floor sealants, polishes or any coatings should not under any circumstances be used on inserts.

Daily Cleaning

Brush down the channel using a soft bristled brush and remove dust particles from the insert by wiping with a damp cloth.

Aluminium

Stubborn marks such as boot blacking may be removed from the aluminium channel by means of a green scotch pad and neutral detergent. Light surface scratches can be removed with careful use of fine wire wool and buffed with a soft cloth.  If the channel has been anodised, wire wool should not be used.

Visual Inspection - Monthly

Check the floor trim for physical damage and for any exposed LED strips or cables.  Check lighting system in floor trims for faulty LEDs (normally indicated by uneven spacing).

Full System Test - Annually

A full system test should be conducted by a competent service engineer/electrician.

Note: a visual inspection may be carried out by responsible personnel but it is recommended that actual maintenance should be carried out by suitably qualified personnel (qualified electrician).

Data Sheet:   (sheet 1 of 1)   19/04/11

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com