Installation Guidelines

# Accessories

PVC Cove Formers

Data Sheet:   (sheet 1 of 1)   March 2015 KRO

These installation guidelines apply to the following Gradus profiles:

CF20, CF32P, CF38

Cove formers are used to form a cove where floorcoverings are continued up the wall.  Cove formers are usually used in conjunction with capping seals or carpet caps.

The cove former should be allowed to acclimatise for 24 hours prior to installation at an ambient temperature of 18o Celsius.

- It should be ensured that all surfaces to which the profile is to be fixed are sound, dry, flat and free from any contaminants.  Wall surfaces should be free from any previous coverings, linings or adhesive residues used to fix them.  Paint films and any mould must be removed and the affected surface treated correctly.

- Lay the lengths of cove former around the perimeter of the area and, using a sharp knife or hacksaw, cut the lengths to size.

- All external and internal corners should be cut with the aid of a mitre block.

- Once cut, remove the profile away from the wall and lay face down.

- Apply one of the recommended adhesives listed below and follow the manufacturers instructions for application.

- Place the profile in position, apply firm pressure along the full length to ensure full contact is made by the profile between the wall and the floor.

Recommended adhesive products for PVC cove formers

Gradus Limited provides this information only as guidance and the legal responsibility for the supply and performance of the adhesives listed below shall remain the sole responsibility of the relevant manufacturer.

By providing this information, Gradus Limited does not imply any equivalents between each of the different manufacturers’ products, or that other products would prove unsatisfactory.  However, use of adhesives not recommended by Gradus should be supported by a guarantee from the adhesive supplier.

Application of the named adhesive must be actioned in strict accordance with the relevant manufacturers’ instructions.

Gradus do not recommend the use of spray adhesives and double sided tapes.

If you have any queries regarding adhesives, please contact the adhesive manufacturer directly.

| Manufacturer | Product Reference | Contact number | Website |
| --- | --- | --- | --- |
| F. Ball and Co. Ltd | F60 / F66 | +44 (0) 1538 361 633 | http://www.f-ball.co.uk/ |
| Tremco | SP060 / SF115 | +44 (0) 1942 251400 | http://www.tremco-illbruck.co.uk/ |
| UZIN | WK 222 / KE 2000 S | +44 (0) 1788 530 080 | http://www.uzin.co.uk/ |

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

08/13/DT